# 8-BIT SUBTRACTION WITH BORROW USING DIRECT ADDRESSING

#### AIM:

To write an assembly language program to subtract tow numbers of 8-bit data stored in memory locations 4200H and 4201H and store the result in 4202H and 4203H with borrow using direct addressing.

APPARATUS REQUIRED:

- 8085 microprocessor kit	1

- Power card	1

- Keyboard	1

#### ALGORITHM:

- Load the second data from memory to the accumulator and move it to the B register.

- Load the first data from memory to the accumulator.

- Subtract the content of B – register from accumulator

- If Carry flag = 0 then jump to step 5 & 6

- Increment C register to count the borrow

- Take two’s complement of the difference

- Store the Difference in memory.

- Move the borrow to accumulator and store in memory.

- Stop.

Output:

###### RESULT:

Thus, an assembly language program for subtraction of two 8-bit numbers with borrow was written, executed and Verified the Result successfully using 8085 kit.