# 8-BIT MULTIPLICATION OPERATIONS USING 8085 MICROPROCESSOR

AIM:

To write an assembly language program to multiply two numbers of 8-bit data stored in memory 4200H	and 4201H and store the product in 4202H and 4203H.

APPARATUS REQUIRED:

- 8085 microprocessor kit	1

- Power card	1

- Keyboard	1

#### ALGORITHM:

- Load the first data in ACC and move to E.

- Load the second data ACC and move to B (count)

- Clear HL pair (Initial sum)

- Clear D for overflow (carry)

- Add the content of DE to HL

- Decrement the count.

- Check whether the count has reached zero.

- Check the zero flag. If ZF = 0, repeat addition or If ZF = 1, go to next step

- Store the content of HL in memory. (Least significant 16 bits of the product)

- Stop.

#### Output:

#### RESULT:

#### Thus, an assembly language program to multiply two numbers of 8-bit data was written, executed and Verified the Result successfully using 8085 kit.

# AIM: